#L44

Q: Can I eliminate the vector “path” association in SET files?

Saving Vectors Without Path

Many users find it advantageous (even crucial at times) to make SET and TRN files portable between folders.   Presently, when a SET file is saved and a vector file is associated with it, the vector file (.TRN) must be located in the same path as when the two files were created.  There is a way to save all files that you create from now on so that this pathing restriction is avoided, and you can also resave SET files you’ve already created and eliminate the path previously stipulated.  That is, the vectors can be saved without a path so that the “Download Vectors” window that appears when opening the setfile looks only in the current

folder for the vectors.   (The same is true for PRG files, by the way)  This requires

a special script to be enabled.  We’ll tell you how to do

that in a moment, but first let’s talk about managing

your Special Script file in general.

You may recall that Special Script is a way that users

can enable certain features in the tester that have not yet

been incorporated into the released software.  The

file “SpScript.txt” presently in your C:\ETS123 folder contains scripts

(unreleased features) that you are currently using.  Actually, it probably

lists all the scripts and simply enables certain ones

by the absence of the leading semicolon at column zero.  Open

this file and look at it.

.

.

.

;NEW FEATURES (Recommended)

;AllowNewBetaFeatures; Generic

;SaveVectorFileWithoutPathIfPathMatchesSetFilePath

DisplayLimitIfDcPmuLimitExceeded

AllowIddqScreen

;AutoTestUsePmuPer32

.

.

.

SpScript.txt Partial Example

Every time you start the ETSNT software, it looks for SpScript.txt in the ETS123 folder. All enabled keywords in SpScript.txt are activated unless you rename the file and restart.

If you don’t see the SpScript.txt file, you can create one.  Simply start ETSNT (it doesn’t

matter if the tester is on or connected at this point).  On

the launchpad, click Translators.  Then click “Special

Scripts” at the bottom of that window.  If you click the

“Scripts...” button, a file is created called

AllScriptNames.txt, containing all the special scripts.

Some scripts (recommended ones) are already enabled.  Save this file as SpScript.txt.

When you receive software updates, there will likely be new Special Scripts.  To take advantage of these while retaining your previously enabled scripts, rename your old SpScript.txt file before performing the above procedure.

Then you can enable the same scripts that

were enabled in your old SpScript.txt file, by removing the

leading semicolons in the AllScriptNames.txt file and saving as SpScript.txt.

To use the vector path feature that is the topic of this Q’nApp, you will enable one more script.  Note in the “partial example” on page one that there is a keyword called:

## SaveVectorFileWithoutPathIfPathMatchesSetFilePath

Remove the semicolon from the front of this keyword and

save this file as SpScript.txt.

Now restart the ETSNT software to enable the Special Script.

This feature will save your vector file without a path as

long as it matches the setfile name.  To do this, save your

setfile by clicking File and then “Save as…”, and enter a new name for the setfile (It is required that your ETS system is on and connected at this point).    Then the Vector Upload window appears:

When you click the red question mark button, you will be asked this question:

Answer “Yes” and you will return to the Vector Upload window.  There you’ll see that your vector filename matches the setfile name… only the extensions are different.  You’ll also notice that the current path is defined with the vector filename:

Click the “Browse…” button, normally used to define a filename and location for the vector file.   In the Browse window, the vector name will already have been defined:

Simply click Save and you’ll return to the Vector Upload window.  Only now the path is no longer indicated:

Just click “Save & Upload” at the bottom of the window and that completes the function.   Any files

saved this way will be transportable between folders.  Use the same procedure if you have .PRG files.

See also:

Q’nApp #L15 on Special Scripts

Q’nApp #L16 on ETSNT files

Q’nApp #L19 on the Little Red Question Mark