#L6

Q: How can I disconnect an assigned pin from the DUT?

HILEVEL Solution to Synchronization and Multiplexing

| We primarily see just two reasons for disconnecting a signal from the DUT. - You wish to use tester pin(s) for synchronization with an external instrument. - You are using Timing-On-the-Fly (TOF) to achieve higher clock rates. In both cases it is important to disconnect pins to avoid misleading failure messages when running DUT Continuity test. In the latter case, the disconnect state is the default when specifying TOF parameters, while the former represents a case where the ETS has no knowledge of the user’s intentions. So in either case we have to explicitly tell the system that the pin in question is not really connected to the DUT. Synchronization Pins The synchronization case is straight forward. You simply define your pin groups, including group(s) used for synchronization. To disconnect the pin you invoke the Pin Setup screen (F2) and press the button “Pins...” which takes you to the dialog box describing all your pins. Simply double click on this pin in the left ListBox; under the column marked Special, the flag DC will then appear, signifying that the pin has been disconnected. | Timing On-the-Fly For the TOF case you decide on connection while defining TOF pins. The TOF screen is invoked from the LaunchPad (Alt+F5 will also do). You there select the number of pins to be multiplexed (two, three or four), and the system automatically disconnects the adjacent pins that are part of the multiplexing. If you indeed wish to connect one of these pins to the DUT anyway, you then select the signal and double click in the List box of the TOF screen. A signal used for Timing-On-the-Fly is flagged as Tof or TofDc, depending on whether the pin is connected to the DUT or not. Listing Disconnected Pins When using the SetFile Viewer, the number of disconnected pins are listed. It provides an easy and convenient integrity check. The SetFile Viewer is found in the View drawer above the toolbar. |
| --- | --- |