#L36

Q: What is the functional description of HiLevel's Memory Test feature?

# Memory Test Functional Description

Stimulus Data has special meaning when the ETS is in Memory Test mode. Each byte of stimulus data (for memory address/data only) controls special functions within the Memory Test circuitry of the Pin Electronics. The specific functions are determined by the selected Algorithm.

## Stimulus Values Pertaining to Memory Test

When referencing the table below, refer also to the Memory Test Functional Diagram.  The following information applies to each byte of stimulus data for both address and data.  Note that stimulus bit 7 functions as a strobe to the memory test circuit and is indicated as Memory Edge in the Pin Setup Window. Edge refers to the 'active' edge of the strobe.

Stimulus		Function	Comments

0x00-0x01		NOOP

0x90		y = CONSTANT	Most commonly used to load/reload initial value used

by the specified Algorithm

0xA0		y = f(x)	Output previous result (effectively a NOOP)

y = f(x*2)             Shift left one bit position, insert '0'

y = f(x/2)	Shift right one bit position, insert '0'

0xA1		y = f(x+1)	Increment

y = f(x*2)	Shift left one bit position, insert '1'

y = f(x/2)	Shift right one bit position, insert '1'

__

0xB0		y = f(x)	Complement.

_____

0xB1		y = f(x) +1

0xC0-0xCF		Load CONSTANT	LS 4 bits are loaded into LS byte of 32-bit						CONSTANT (bits 0-27 of CONSTANT

are left-justified).

NOTE:  The specific FUNCTION (increment, shift) is determined by the selected Algorithm. The input vectors have direct control over carry/serial input, complement, loading of CONSTANT register only.

Bit 7 of each stimulus byte is an RZ stimulus bit. This stimulus bit clocks the FUNCTION results into the OUTPUT register of the memory test circuit and loads/shifts data into CONSTANT, four bits at a time. The trailing  (clocking) edge of this stimulus bit is defined by Memory Edge in Pin Setup.

Under normal operation, vector data is applied to the DUT exactly as defined in Pin Setup. However, when Memory Test is selected, vector data controls special functions used in the generation of specific patterns which are determined by selected algorithms. Therefore, the expected response data reflects the response expected as a result of the applied stimulus from the Memory Test circuit and not the Vector Stimulus. For example, if a Checkerboard pattern is chosen under Algorithms in Memory Test Setup, the expected response will be either 0x55 or 0xAA, while the vector stimulus will appear as previously defined under Stimulus Values Pertaining to Memory Test. In order to ease confusion and facilitate debug/verification, a special 'Pin Direction' is automatically selected in Pin Setup. 'Split Memadr' and 'Split Memdata' allow the user to view (in the Analysis screen) Stimulus Data both as it is input to the Pin Electronics (to Memory Test), and as it is input to the DUT (from Memory Test) within the same test cycle.

The diagram below applies both to memory address and memory data. It is repeated for each byte of address or data. Also, see Example 1 on the next page.

### Memory Test Functional Diagram

Memory Test Vectors

00005-	Loads '00000000' into 32-bit CONSTANT for each byte of address stimulus.

0000C	Loads '55555555' into 32-bit CONSTANT for data stimulus.

0000D	Loads OUTPUT Registers with CONSTANTS for address and data stimuli.

(i.e. address='0000', data='55' - See NOTE)

0000E-	Performs no operation relative to memory stimulus. These vectors set up

00010	the loop count in preparation for generating memory write stimulus.

00011	This vector increments the address and complements the data stimulus for each test cycle (determined by Memory Edge timing). The test loops on this vector (decrementing the loop count each cycle), until the count reaches zero.

00012	Loads OUTPUT Registers with CONSTANTS (i.e. address='0000', data='55')

00013-	No operation for memory stimulus. Sets up loop count in preparation for generating

00014	memory read stimulus. Note that the loop count reflects the fact that there are 2 memory read cycles for each decrement of the count.

00015	Compares memory data (from DUT) with expected data '55' and increments the address stimulus. The loop count is decremented.

00016	Compares memory data (from DUT) with expected data 'AA' and increments the address stimulus. If the loop count has not decremented to zero, vector 00015 is repeated.

NOTE:	Although only 15 bits of address and 8 bits of data are significant, all functions are performed with 32-bit operands.

Example 1

Also See:  Q'nApp #L1: PinList				© 1999 HILEVEL Technology Inc.

Q'nApp #L4: Using Memory Test